2U User Manual

V5.2

TABLE OF CONTENTS

Chapter 1	Specifications	11

Chapter 2	Overview	13

2.1	Front View	13

2.1.1	LFF Model	13

2.1.2	SFF Model	14

2.2	Rear View	15

2.3	Power Supply (PSU) View	16

Chapter 3	Installation / Removal	17

3.1	System Components	17

3.1.1	Hard Drive Installation (for SFF & LFF)	17

3.1.2	Hard Drive Removal (for SFF & LFF)	18

3.1.3	SFF Hard Drive Carrier Removal	18

3.1.4	LFF Hard Drive Carrier Removal	19

3.1.5	PSU Removal	20

3.1.6	PSU Installation	21

3.1.7	Memory Installation	22

3.1.8	Memory Removal	23

3.1.9	CPU Installation	24

3.1.10	CPU Removal	26

3.1.11	CPU Heatsink Installation	27

3.1.12	CPU Heatsink Removal	28

Chapter 4	BIOS Setup	31

4.1	Overview	31

4.1.1	Main	31

4.1.2	Advanced	33

4.1.3	iSCSI Configuration	34

4.1.4	Intel(R) Virtual RAID on CPU	35

4.1.5	Trusted Computing	36

4.1.6	Serial Port Console Redirection	37

4.1.7	Console Redirection Settings	38

4.1.8	Option ROM Dispatch Policy	39

4.1.9	PCI Subsystem Settings	41

4.1.10	Network Stack	42

4.1.11	CSM Configuration	43

4.1.12	NVMe Configuration	44

4.1.13	IntelRCSetup	45

4.1.14	PCH SATA Configuration	46

4.1.15	PCH sSATA Configuration	48

4.1.16	USB Configuration	50

4.1.17	Runtime Error Logging	51

4.1.18	Socket Configuration	52

4.1.19	Processor Configuration	53

4.1.20	Advanced Power Management Configuration	55

4.1.21	Hardware PM State Control	57

4.1.22	Memory Configuration	59

4.1.23	IIO Configuration	61

4.1.24	Server Mgmt	64

4.1.25	System Event Log	65

4.1.26	BMC network configuration	66

4.1.27	BMC network configuration	68

4.1.28	Security	69

4.1.29	Boot	70

4.1.30	Save & Exit	71

Appendix A 	Motherboard connectors and slots	72

Appendix B     Memory Slot Layout	73

Appendix C	 Memory Module Installation Order	74

Appendix D	 BIOS Function Key	75

Appendix E	 LED Descriptions	75

Safety and Warnings

CAUTION:

Before installing and starting up a device, please observe the safety instructions listed in the following sections. This will help you to avoid making serious errors that could impair your health, damage the device and endanger the data base.

CAUTION:

To reduce the risk of electric shock, this equipment must be installed by trained service personnel in a restricted-access location.

CAUTION:

When laser products (such as CD-ROMs, DVD drives, fiber optic devices, or transmitters) are installed, note the following:

Do not remove the covers. Removing the covers of the laser product could result in exposure to hazardous laser radiation.

There are no serviceable parts inside the device.

Use of controls or adjustments or performance of procedures other than those specified herein might result in hazardous radiation exposure.

CAUTION:

Connect all power cords to a properly wired and grounded electrical outlet.The power control button on the device and the power switch on the power supply do not turn off the electrical current supplied to the device. The device also might have more than one power cord. To remove all electrical current from the device, ensure that all power cords are disconnected from the power source.

CAUTION:

Incorrectly installing a battery or using an incompatible battery may increase the risk of fire or explosion. Replace the battery only with the same or equivalent type.

Do not:

Throw or immerse into water.

Heat to more than 100°C (212°F).

Repair or disassemble.

Dispose of the battery as required by local ordinances or regulations.

CAUTION:

The following label indicates a hot surface nearby.

CAUTION:

Hazardous energy is present when the server is connected to the power source. Always replace the blade cover before installing the server.

CAUTION:

Hazardous moving parts are nearby.

CAUTION:

Slide/rail mounted equipment is not to be used as a shelf or work space. Do not add weight to slide/rail mounted equipment.

Installation Safety Information

Preparing for startup

The devices comply with the relevant safety regulations for information technology equipment.

The requirements, which need to be fulfilled at the site of installation, are described in the user documentation for this device. Please contact the service center if there is any doubt as to the safety of installing the device at the intended site.

Transporting, unpacking, installing

Condensation may form when the device is brought into the operations room from a colder environment. Wait until the device has warmed up to the room temperature and is totally dry before starting it. The acclimation time depends on the device and its design.

Connecting data cables

CAUTION:

No data transmission lines may be connected or disconnected during a storm (danger of being struck by lightning).

When wiring the devices, the cables need to be connected or removed in the order described in the user document for the device. When connecting or disconnecting any of the leads, always hold them by the plug. Never pull on the cables themselves. Doing so could cause a cable to become detached from the plug.

Connecting the system to the power mains

Please check devices with adjustable rated voltage to determine whether the preset rated voltage of the device conforms to the local mains voltage. An incorrect setting leads to damage to or destruction of the device.

Before operating, check whether all the cables and wires are in perfect, undamaged condition. Ensure in particular that the cables have not been bent, have not been laid too tightly round corners, and that there are no objects located on top of them. Also make sure that all connectors have a tight fit. Defective screening or wiring may damage your health (electric shock) and can damage other devices.

Devices with power plugs are equipped with a safety-tested AC power line of the country of use and may only be connected to an approved shock-proof socket. This may otherwise result in an electric shock.

Safety during operation

Avoiding short circuits

Make sure that no objects (e.g. jewelry, paper clips, etc.) or liquids get inside the device. This can lead to electric shocks or short-circuits.

Ventilation slots

Please make sure that the air vents are not blocked or collect dust as this may lead to the risk of overheating while the device is in operation. This could lead to operating faults.

Proper operation

Proper operation and compliance with the EMC (electromagnetic compatibility) limit values is only guaranteed when the housing cover is mounted correctly and the doors are closed (cooling, fire protection, screening against electrical, magnetic, and electromagnetic fields).

Switch off in the event of malfunction and during servicing

Devices are not disconnected from the mains by simply switching them off.

In the event of a malfunction or servicing, the devices need to be disconnected from the mains immediately.

Please proceed as follows:

Switch off the devices,

Pull out the mains plug (also refer to the device's user documentation),

Inform Service.

Devices that are connected to one or more uninterruptible power supplies (UPS's) will continue to operate even if the plug to the UPS ('s) is pulled. You therefore need to shut down the UPS ('s) in accordance with the accompanying user documentation.

Maintenance Safety

Expanding, repairing

When expanding the device, use only parts that have been approved for the device. Failure to observe this rule can lead to violation of the electromagnetic compatibility (EMC) or safety standards and cause device malfunctions.

The device may only be repaired by authorized, qualified personnel. Improper repairs may expose the user to considerable danger (electric shock, fire).

Unauthorized opening of the device or individual parts of the device can also expose the user to considerable danger. Unauthorized opening of the devices or parts thereof results in voiding of the warranty and exclusion of liability.

Handling batteries

The life of the batteries/accumulators in the devices is approx. three to five years. In order to ensure the functional reliability, they must be exchanged at the end of this time. The batteries may only be changed by authorized personnel. The local regulations for disposal of special waste must be observed when disposing of the batteries.

Batteries can cause danger, e.g. fire, if handled incorrectly. Therefore avoid opening, puncturing or pressing together batteries. Never throw batteries on a fire.

Special safety note for rack cabinets

Do not use device units mounted on pull-out rails as a surface on which to put things or as a work surface, and strictly avoid leaning on or against them.

Setting up a rack

At least two people must always be used to set up a rack because of its weight and their size.

This is the only way to avoid accidents and damage to the equipment.

To install the server in the rack cabinet, please observe the instructions in the relevant system installation manual.

Overload protection

Make sure if connecting a number of devices to the same circuit that you do not overload the current distribution. Please observe the nominal values indicated on the product ident plates.

Stabilizing the racks

Even when the rack has been secured against tipping over, only one slide-in module may be removed on its rails at any one time. There is no guarantee that the rack will remain stable if several modules are pulled out simultaneously.

Second person for work on racks

Two or more people are required to insert or remove rack trays as these are large and heavy.

This is particularly true regarding servers, peripheral devices and UPS's. This information can be found in the device's user documentation.

WEEE

The device may not be disposed of with household rubbish. This appliance is labelled in accordance with European Directive 2002/96/EG concerning used electrical and electronic appliances (waste electrical and electronic equipment – WEEE).

The guideline determines the frame-work for the return and recycling of used appliances as applicable throughout the EU. To return your used device, please use the return and collection systems available to you.

Regional EMC Compliance Information

FCC Verification Notice (USA only)

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

(1) this device may not cause harmful interference, and

(2) this device must accept any interference received, including interference that may cause undesired operation.

Class A

This equipment has been tested and found to comply with the limits for a Class A digital device pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment.

This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the manufacturer’s instruction manual, may cause harmful interference with radio communications. Operation of this equipment in a residential area is likely to cause harmful interference, in which case you will be required to correct the interference at your own expense.

INDUSTRY CANADA (Canada only)

This Class B (or Class A, if so indicated on the registration label) digital apparatus meets the requirements of the Canadian Interference-Causing Equipment Regulations.

Cet appareil numérique de la Classe B (ou Classe A, si ainsi indiqué sur l’étiquette  d’enregistration) respecte toutes les exigences du Reglement sur le Materiel Brouilleur du Canada.

CE Declaration of Conformity (EUROPE only)

This product has been tested in accordance to, and complies with the European Low Voltage Directive (73/23/EEC) and European EMC Directive (89/336/EEC).

The product has been marked with the CE Mark to illustrate its compliance.

中国CCC (China only)

The following CCC EMC Warning is marked on the product: EMC Warning is required for Class A products.

此为A级产品，在生活环境中，该产品可能会造成无线电干扰，在这种情况下，可能需要用户对其干扰采取可行的措施

China RoHS Declaration Table

| 部件名称 | 有害物质 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 铅(Pb) | 汞(Hg) | 镉(Cd) | 六价铬(Cr(VI)) | 多溴联苯(PBB) | 多溴二苯醚(PBDE) |
| PCB板 | × | ○ | ○ | ○ | ○ | ○ |
| 结构件 | ○ | ○ | ○ | ○ | ○ | ○ |
| 芯片及 其他主动零件 | × | ○ | ○ | ○ | ○ | ○ |
| 连接器 | × | ○ | ○ | ○ | ○ | ○ |
| 风扇、散热件 | ○ | ○ | ○ | ○ | ○ | ○ |
| 硬盘 | ○ | ○ | ○ | ○ | ○ | ○ |
| 助焊剂，散热膏，标签及其他耗材 | ○ | ○ | ○ | ○ | ○ | ○ |
| 本表格依据 SJ/T 11364 的规定编制。 ○：表示该有害物质在该部件所有均质材料中的含量均在 GB/T 26572 规定的限量要求以下。 ×：表示该有害物质至少在该部件的某一均质材料中的含量超出 GB/T 26572 规定的限量要求。 注：表中标记“×”的部件，皆因全球技术发展水平限制而无法实现有害物质的替代。 |  |  |  |  |  |  |

Specifications

| Form Factor | 2U 19” Rack-mount Chassis dimension: 735mm (L) x 438mm (W) x 87mm (H) |
| --- | --- |
| Platform | Intel® Xeon® scalable processor family, up to 165W *Note1 Intel Lewisburg PCH C621 |
| CPU | 2nd Generation Intel® Xeon® Scalable Processors (Cascade Lake-SP), Intel® Xeon® Scalable Processors., Dual Socket LGA-3647 (Socket P) supported Up to 205W(MB support) |
| Rear I/O | 1 * Dedicated Management RJ45 port 1 * VGA port 1 * UID (Reset) Button with LED 1 * Power Button with LED 2 * USB 3.0 1 * COM Port |
| Memory | 24 * DDR4 DIMM slots Support DDR4 RDIMM/LRDIMM ECC & DCPMM (2933 MT/s) Support 8GB/16GB/32GB/64GB/128GB DIMM |
| Internal I/O | 7 * PCIe Gen3 x8 or 2 * PCIe Gen3 x16 + 3 * PCIe Gen3 x8 2 * M.2 (Type 2280) 1 * OCP Slot 1 * TPM 2.0 (Optional) |
| Power Supply | 2 * Hot Swap PSU 550/800W (Redundancy by system configuration) |
| Storage | Front: - 12 * 3.5” SATA/SAS HDD - 24 * 2.5” SATA/SAS HDD - 12 * 3.5” SATA/SAS (4* NVMe) HDD Rear: - 2 * 2.5” SATA Support RAID 0, 1, 5, 10 |
| Cooling | 6 * 6038 fans, hot swap |
| Front I/O and LED | 1 * Power Button with LED 1 * UID Button with LED 1 * Reset Button 1 * HDD LED 1 * System Status LED 1 * LAN LED 1 * VGA ports 2 * USB 3.0 |
| Operating Temperature | 10°C to 35°C |

*Note1 Depend on shipping configuration, it’s required to apply 2U heatsink for CPU over 150W. For detail, please contact Sales representative.

Overview

Front View

LFF Model

| Number | Component |
| --- | --- |
| 1 | VGA port |
| 2 | USB 3.0 |
| 3 | Service tag |
| 4 | 12 x HDD (0 ~ 11) |
| 5 | USB 3.0 Port |
| 6 | Power / UID / Reset Button |
| 7 | LED (HDD status / System Status/ LAN1) |

SFF Model

| Number | Component |
| --- | --- |
| 1 | VGA Port |
| 2 | USB 3.0 |
| 3 | Service Tag |
| 4 | 24 x HDD (0~23) |
| 5 | USB 3.0 |
| 6 | Power / ID /Reset Button |
| 7 | LED (HDD status / System Status/ LAN1) |

Rear View

| Number | Component |
| --- | --- |
| 1 | Power Supply x 2 |
| 2 | COM Port |
| 3 | VGA Port |
| 4 | RJ45 Management Port |
| 5 | UID LED |
| 6 | Power LED |
| 7 | USB 3.0 x 2 |
| 8 | OCP Slot |
| 9 | Slot 0 (PCe x 8 or PCIe x16/CPU_1) |
| 10 | Slot 1 (PCe x 8/CPU_1) or N.A (When Slot 0 PCIe x16 present) |
| 11 | Slot 2 (PCe x 8/CPU_2) |
| 12 | Slot 3 (PCe x 8 or PCIe x16/CPU_2) |
| 13 | Slot 4 (PCe x 8/CPU_2) or N.A (When Slot 3 PCIe x16 present) |
| 14 | Slot 5 (PCe x 8/CPU_2) |
| 15 | Slot 6 (PCe x 8/CPU_1) |
| 16 | 2.5” HDD x 2 |

Power Supply (PSU) View

| Number | Component |
| --- | --- |
| 1 | PSU Handle - Pull to remove the PSU |
| 2 | Power Connector - Connects to a power source |
| 3 | PSU LED Indicator - Indicates the status of the power supply |
| 4 | PSU Release Latch - Press to release the PSU from the system |
| 5 | PSU Fan – Cool fan of the PSU unit |

Installation / Removal

System Components

Hard Drive Installation (for SFF & LFF)

1.	Remove the hard drive carrier from the system.

2.	Remove the hard drive from the hard drive carrier. 
Skip this step if the hard drive carrier does not have a hard drive installed.

3.	Slide the hard drive into the hard drive carrier ensuring the positioning studs fit into the holes at the side of the hard drive.

4.	Press down on the hard drive to secure the hard drive to the carrier.

Hard Drive Removal (for SFF & LFF)

1.	Remove the hard drive carrier from the system.

2.	Push outwards the hard drive release latches on the hard drive carrier while pressing down on the hard drive to remove the hard drive from the hard drive carrier.

SFF Hard Drive Carrier Removal

- Press the carrier release handle button then pull down the carrier release handle.

- Pull the carrier release handle to remove the hard drive carrier.

LFF Hard Drive Carrier Removal

- Press the carrier release handle button then pull down the carrier release handle.

- Pull the carrier release handle to remove the hard drive carrier.

PSU Removal

- Pull the PSU handle to a 90-degree angle.

2.	Press the PSU release latch while pulling the PSU handle to remove the PSU.

PSU Installation

- Push the PSU to end of the slot and make sure the latch is locked.

- Push the PSU handle to a 0-degree angle. (See figure below)

Memory Installation

- Remove the air duct from the system.

- Insert memory DIMM to slot, ensure module notch is aligned with slot key.

- Push the retention clips inwards to lock the memory DIMM.

- Install the air duct.

Memory Removal

1.	Remove the air duct from the system.

2.	Push the retention clips outwards to release the memory DIMM.

3.	Lift and remove the memory DIMM.

IMPORTANT:	The server requires at least one DIMM per processor for functioning. When you install/remove memory DIMMs, please follow the sequence shown in the table (Appendix B & C) to maintain performance.

CPU Installation

- Remove the air duct from the chassis.

- Remove either the CPU heatsink 1 or CPU heatsink 2 from the chassis.

- Remove the CPU from the CPU carrier.

- Slide the CPU into the CPU carrier ensuring that the triangle mark on the CPU is aligned with the triangle mark on the CPU carrier, and then secure the CPU to the CPU carrier.

- Install the CPU carrier onto the CPU heatsink. Ensure that the triangle mark on the CPU carrier is aligned with the blunt corner of the CPU heatsink and that the CPU carrier is firmly fixed on the CPU heatsink by the latches.

CPU Removal

1.	Remove the air duct from the chassis.

2.	Remove either the CPU heatsink 0 or CPU heatsink 1 from the chassis.

3.	Carefully push the CPU carrier tab outwards to release the CPU from the CPU carrier.

4.	Lift to remove the CPU from the CPU carrier.

CPU Heatsink Installation

- Remove the air duct from the chassis.

- Put the CPU heatsink on socket.

- Tighten the 4 captive screws in sequential order based on the numbers indicated on the heatsink (1 > 2 > 3 > 4)

IMPORTANT: 	When installing the CPU heatsink, follow the sequence indicated on the heatsink. Incorrect sequence will result in an uneven application of the thermal grease on the CPU.

CPU Heatsink Removal

- Remove the air duct from the system.

- Loosen the 4 captive screws in sequential order based on the numbers indicated on the heatsink (4 > 3 > 2 > 1).

4.	Lift and remove CPU heatsink.

IMPORTANT: 	When removing the CPU heatsink, follow the sequential order based on the numbers indicated on the heatsink. Incorrect sequence will result in an uneven application of the thermal grease on the CPU.

BIOS Setup

Overview

This document provides the information for Basic Input-Output System (BIOS) and software features. It describes the architecture and feature set for all function in the system.

Main

| Item | Option | Description |
| --- | --- | --- |
| UEFI Firmware Version | Read only | Display BIOS version |
| Build Date and Time | Read only | Display System BIOS Build Date and Time |
| BMC Firmware Version | Read only | Display BMC version |
| Platform Information |  |  |
| Platform | Read only | Display Platform name |
| Processor Version | Read only | Display Processor name |
| Processor Stepping | Read only | Display Processor Stepping |
| Processor Microcode | Read only | Display Processor Microcode |
| PCH | Read only | Display PCH name |
| RC Revision | Read only | Display RC Revision |
| ME Firmware Version | Read only | Display ME Firmware Version |
| ME State | Read only | Display ME State |
| Memory Information |  |  |
| Total Memory | Read only | Display total memory size |
| Memory Topology |  |  |
| System Date | DAY MM/DD/YYYY | Set system date |
| System Time | XX:XX:XX | Set system time |

Advanced

| Item | Option | Description |
| --- | --- | --- |
| iSCSI Configuration |  | See the table below |
| Intel(R) Virtual RAID on CPU |  | See the table below |
| Trusted Computing |  | See the table below |
| Serial Port Console Redirection |  | See the table below |
| Option ROM Dispatch Policy |  | See the table below |
| PCI Subsystem Settings |  | See the table below |
| Network stack Configuration |  | See the table below |
| CSM Configuration |  | See the table below |
| NVMe Configuration |  | See the table below |
| Wake system from S5 | [Disabled] Enabled | Enable/Disable Wake system from S5 |

iSCSI Configuration

Intel(R) Virtual RAID on CPU

Trusted Computing

| Item | Option | Description |
| --- | --- | --- |
| Security Device Support | Disabled [Enabled] | Enables or Disables BIOS support for security device. O.S. will not show Security Device. TCG EFI protocol and INT1A interface will not be available. |
| Active PCR banks | Read only |  |
| Available PCR banks | Read only |  |
| Pending operation | [None] TPM Clear | Schedule an Operation for the Security Device. NOTE: Your Computer will reboot during restart in order to change State of Security Device. |
| Platform Hierarchy | Disabled [Enabled] | Enable or Disable Platform Hierarchy |
| Storage Hierarchy | Disabled [Enabled] | Enable or Disable Storage Hierarchy |
| Endorsement Hierarchy | Disabled [Enabled] | Enable or Disable Endorsement Hierarchy |
| TPM2.0 UEFI Spec Version | TCG_1_2 TCG_2 | Select the TCG2 Spec Version Support,TCG_1_2: the Compatible mode for Win8/Win10,TCG_2: Support new TCG2 protocol and event format for Win10 or later |
| Physical Presence Spec Version | 1.2 1.3 | Select to Tell O.S. to support PPI Spec Version 1.2 or 1.3. Note some HCK tests might not support 1.3. |
| TPM 20 Interface Type | CRB TIS | Select the Communication Interface to TPM 20 Device. |

Serial Port Console Redirection

| Item | Option | Description |
| --- | --- | --- |
| Console Redirection | Disabled [Enabled] | Console Redirection Enable or Disable. |
| Console Redirection Settings |  | See the table below |

Console Redirection Settings

| Item | Option | Description |
| --- | --- | --- |
| Terminal Type | VT100 VT100+ VT-UTF8 [ANSI] | The VT100 and VT100+ terminal emulations are essentially the same. VT-UTF8 is a UTF8 encoding ofVT100+. PC-ANSI is the native character encoding used by PC-compatible applications and emulators. |
| Bit per second | 9600 19200 38400 57600 [115200] | Serial port transmission speed. This setting must match the remote terminal application. |
| Data Bits | 7 [8] | Serial port transmission data bits. |
| Parity | [None] Even Odd Mark Space | A parity bit can be sent with the data bits to detect some transmission error. Even: parity bit is 0 if the num of 1’s in the data bits is even. Odd: parity bit is 0 if num of 1’s in the data bits is odd. Mark: parity bit is always 1. Space: parity bit is always 0. Mark and Space parity do not allow for error detection. |
| Stop Bits | [1] 2 | Stop bits indicate the end of a serial data packet. (A start bit indicates the beginning.)The standard setting is 1 stop bit. Communication with slow devices may require more than 1 stop bit. |
| Flow Control | [None] Hardware RTS/CTS | Flow control can prevent data loss from buffer overflow. |
| VT-UTF8 Combo Key Support | Disabled [Enabled] | Enable VT-UTF8 Combination Key Support for ANSI/VT100 terminals |
| Recorder Mode | [Disabled] Enabled | With this mode enabled only text will be sent. This is to capture Terminal data. |
| Resolution 100x31 | [Disabled] Enabled | Enables or disables extended terminal resolution |
| Putty KeyPad | [VT100] LINUX XTERMR6 SCO ESCN VT400 | Select Function Key and KeyPad on Putty. |

Option ROM Dispatch Policy

| Item | Option | Description |
| --- | --- | --- |
| Restore if Failure | Disabled [Enabled] | If system fails to boot and this option is set to 'Enabled', software will reset settings of this page as well as CSM page to its default values automatically. |
| Primary Video Ignore | Disabled [Enabled] | If software will detect that due to the Policy settings, Option ROM of Primary Video Device will not dispatch, it will ignore this device policy settings, and restore it to 'Enable' automatically. |
| Device Group Default ROM Policy (Select at CSM Setup Page) |  |  |
| Network Class | Read only |  |
| Mass Storage Class | Read only |  |
| Display Class | Read only |  |
| Other Devices | Read only |  |
| Device Class Option ROM Dispatch Policy: | Read only |  |
| On Board Mass Storage | Disabled [Enabled] | Onboard Device Has: UEFI[X] Legacy[X] Embedded ROM(s). VIDx8086;DIDxA1D2 @ s0¦Bx0¦Dx11¦Fx5 |
| On board Mass Storage | Disabled [Enabled] | Onboard Device Has: UEFI[X] Legacy[X] Embedded ROM(s). VIDx8086;DIDxA182 @ s0¦Bx0¦Dx17¦Fx0 |
| On board Display Controller | Disabled [Enabled] | Onboard Device Has: UEFI[X] Legacy[X] Embedded ROM(s). VIDx102B;DIDx552 @ s0¦Bx3¦Dx0¦Fx0 |
| Slot #2 Empty | Disabled [Enabled] | Enable or Disable Option ROM execution for selected slot. |
| Slot #3 Empty | Disabled [Enabled] | Enable or Disable Option ROM execution for selected slot. |
| Slot #4 Empty | Disabled [Enabled] | Enable or Disable Option ROM execution for selected slot. |
| Slot #5 Empty | Disabled [Enabled] | Enable or Disable Option ROM execution for selected slot. |
| Slot #6 Empty | Disabled [Enabled] | Enable or Disable Option ROM execution for selected slot. |
| Slot #7 Empty | Disabled [Enabled] | Enable or Disable Option ROM execution for selected slot. |

PCI Subsystem Settings

| Item | Option | Description |
| --- | --- | --- |
| PCI Bus Driver Version | A5.01.12 |  |
| PCI Devices Common settings: |  |  |
| Above 4G Decoding | Disabled [Enabled] | Enables or Disables 64bit capable Devices to be Decoded in Above 4G Address Space (Only if System Supports 64 bit PCI Decoding). |
| SR-IOV Support | Disabled [Enabled] | Enable or disable the SR-IOV support |

Network Stack

The Network stack Module is set of drivers that produce network protocols.

The UEFI Network Stack implements the TCP/IP network interfaces such as SNP, MNP, ARP, IP, UDP, DHCP, MTFTP, TCP and VLAN.

| Item | Option | Description |
| --- | --- | --- |
| Network Stack | Disabled [Enabled] | Enable / Disable UEFI Network Stack setting. |
| Ipv4 PXE Support | Disabled [Enabled] | Support IPV4 |
| Ipv4 HTTP Support | [Disabled] Enabled | Enable/Disable IPv4 HTTP boot support. If disabled, IPv4 HTTP boot support will not be available. |
| Ipv6 PXE Support | Disabled [Enabled] | Support IPV6 |
| Ipv6 HTTP Support | [Disabled] Enabled | Enable/Disable IPv6 HTTP boot support. If disabled, IPv6 HTTP boot support will not be available. |
| IP6 Configuration Policy | [Automatic] Manual | Set IP6 Configuration Policy |
| PXE boot wait time | 0 | Wait time to press ESC key to abort the PXE boot |
| Media detect count | 1 | Number of times presence of media will be checked |

CSM Configuration

| Item | Option | Description |
| --- | --- | --- |
| Network | [UEFI] Legacy | Select OPRom execute mode of Network type. |
| Storage | [UEFI] Legacy | Select OPRom execute mode of Storage type. |
| Video | [UEFI] Legacy | Select OPRom execute mode of Video type. |
| Other PCI devices | [UEFI] Legacy | Select OPRom execute mode of Other PCI. |

NVMe Configuration

IntelRCSetup

| Item | Option | Description |
| --- | --- | --- |
| PCH SATA Configuration |  | See the table below |
| PCH sSATA Configuration |  | See the table below |
| USB Configuration |  | See the table below |
| Runtime Error Logging |  | See the table below |

PCH SATA Configuration

| Item | Option | Description |
| --- | --- | --- |
| SATA Controller | Disable [Enable] | Enable or Disable SATA Controller |
| Configure SATA as | [AHCI] RAID | Select the SATA or sSATA ports configure type. |
| SATA Port 0 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 1 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 2 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 3 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 4 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 5 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 6 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 7 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |

PCH sSATA Configuration

| Item | Option | Description |
| --- | --- | --- |
| sSATA Controller | Disable [Enable] | Enable or Disable sSATA Controller |
| Configure SATA as | [AHCI] RAID | Select the SATA or sSATA ports configure type. |
| SATA Port 0 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 1 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 2 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 3 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 4 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |
| SATA Port 5 |  | Information Only |
| Port | Disabled [Enabled] |  |
| Hot Plug | [Disabled] Enabled | Designates this port as Hot Pluggable. |

USB Configuration

| Item | Option | Description |
| --- | --- | --- |
| USB HS Physical Connector port 1 | Disabled [Enabled] | Enable/Disable this USB Physical Connector (physical port). Once disabled, any USB devices plug into the connector will not be detected by BIOS or OS. |
| USB HS Physical Connector port 2 | Disabled [Enabled] | Enable/Disable this USB Physical Connector (physical port). Once disabled, any USB devices plug into the connector will not be detected by BIOS or OS. |

Runtime Error Logging

| Item | Option | Description |
| --- | --- | --- |
| System Errors | Disabled [Enabled] | Enable or disable system error reporting and logging in runtime. |
| PCIE Corrected Error Threshold | 3f | Specifies how many errors must occur before triggering the logging of a SEL Error Event. Set value range is 1 - 255, 0 is no threshold. Not available when Runtime Error Logging disabled. |

Socket Configuration

| Item | Option | Description |
| --- | --- | --- |
| Processor Configuration |  | See the table below |
| Advanced Power Management Configuration |  | See the table below |
| Memory Configuration |  | See the table below |
| IIO Configuration |  | See the table below |

Processor Configuration

| Item | Option | Description |
| --- | --- | --- |
| Core Disable Bitmap(Hex) | 0 | 0: Enable all cores. The value should be less than “Dynamic detect number”. Maximum Core number: ”Dynamic detect number”. (Note) |
| Core Disable Bitmap(Hex) | 0 | 0: Enable all cores. The value should be less than “Dynamic detect number”. Maximum Core number: ”Dynamic detect number”. (Note) |
| Hyper-Threading [ALL] | Disabled [Enabled] | Intel® Hyper-Threading Technology allows multithreaded software applications to execute threads in parallel within each processor. Software method to enable or disable logical process or threads. |
| Allow mixed freq among CPUs | [Disabled] Enabled | Keep (mixed) power-on freq of each CPU socket. Caution: This is for Intel PPV test only |
| Check CPU BIST Result | Disabled [Enabled] | Disable failed BIST core when enable, otherwise, ignore BIST result |
| Execute Disable Bit | Disabled [Enabled] | When disabled, forces the XD feature flag to always return 0. |
| Enable Intel(R) TXT | [Disabled] Enabled | Enables Intel(R) TXT. |
| VMX | Disabled [Enabled] | Enables the Vander pool Technology, takes effect after reboot. |
| Enable SMX | [Disabled] Enabled | Use the ENCRYPT_SMX configuration parameter to set the level of encryption for high availability configurations. |
| Lock Chipset | Disabled [Enabled] | Lock or Unlock chipset |
| Hardware Prefetcher | Disabled [Enabled] | Hardware Prefetcher is normally enabled, for best efficiency in L2 Cache and Memory Channel use but disabling it may improve performance for some processing loads and on certain benchmarks. |
| Adjacent Cache Prefetch | Disabled [Enabled] | Adjacent Cache Prefetcher is normally enabled, for best efficiency in L2 Cache and Memory Channel use but disabling it may improve performance for some processing loads and on certain benchmarks. Enabled - Fetches adjacent cache line (128 bytes)when required data is not currently in cache. Disabled - Only fetches cache line with data required by the processor (64 bytes). |
| DCU Streamer Prefetcher | Disabled [Enabled] | The next cache line will be prefetched into L1 data cache from L2 or system memory during unused cycles if it sees that the processor core has accessed several bytes sequentially in a cache line as data.DCU Steamer Prefetcher is normally enabled, for best efficiency in L1 Data Cache and Memory Channel use but disabling it may improve performance for some processing loads and on certain benchmarks. |
| DCU IP Prefetcher | Disabled [Enabled] | The next cache line will be prefetched into L1instruction cache from L2 or system memory during unused cycles if it sees that the processor core has accessed several bytes sequentially in a cache line as data. DCU IP Prefetcher is normally enabled, for best efficiency in L1 Instruction Cache and Memory Channel use but disabling it may improve performance for some processing loads and on certain benchmarks. |
| AES-NI | Disabled [Enabled] | Enable or disable Intel® Advanced Encryption Standard New Instruction (AES-NI) support. |

Advanced Power Management Configuration

| Item | Option | Description |
| --- | --- | --- |
| Speedstep (Pstates) | Disabled [Enabled] | Enable/Disable EIST (P-States) |
| EIST PSD Function | [HW_ALL] SW_ALL SW_ANY | Choose HW_ALL/SW_ALL/SW_ANY in _PSD return |
| Turbo Mode | Disabled [Enabled] | Enable/Disable processor Turbo Mode (requires EMTTM enabled too). |
| Hardware PM State Control |  |  |
| ENERGY_PERF_BIAS_CFG mode | Performance [Balanced Performance] Balanced Power Power | Use input from ENERGY_PERF_BIAS_CONFIG mode selection. PERF/Balanced Perf/Balanced Power/Power |
| CPU C State Control |  |  |
| Enhanced Halt State (C1E) | Disabled [Enabled] | Core C1E auto promotion Control. Takes effect after reboot. |
| Package C State | C0/C1 state C2 state C6(non Retention) state C6(Retention) state No Limit Auto | Package C State limit |
| CPU C6 report | Disabled Enabled [Auto] | Enable/Disable CPU C6(ACPI C3) report to OS |

Hardware PM State Control

| Item | Option | Description |
| --- | --- | --- |
| Hardware P-States | Disabled [Native Mode] Out of Band Mode Native Mode with No Legacy Support | Disable: Hardware chooses a P-state based on OS Request (Legacy P-States) Native Mode: Hardware chooses a P-state based on OS guidance Out of Band Mode: Hardware autonomously chooses a P-state (no OS guidance) |
| HardwarePM Interrupt | [Disabled] Enabled | Enable/Disable Hardware PM Interrupt |
| EPP Enable | Disabled [Enabled] | When disabled, HW masks EPP in CPUID[6].10 and uses EPB for EPP |
| APS rocketing | [Disabled] Enabled | Enable/Disable the rocketing mechanism in the HWP p-state selection pcode algorithm. Rocketing enables the core ratio to jump to max turbo instantaneously as opposed to a smooth ramp up |
| Scalability | [Disabled] Enabled | Enable/Disable the use of scalability in HWP pcode power efficiency algorithms. Scalability is the measure of estimated performance improvement for a given increase in core frequency |
| PPO-Budget | [Disabled] Enabled | Enable/disable core parameter based per core power budgeting. PPO-Budget allocates power budget to cores based on their scalability/EPP |

Memory Configuration

| Item | Option | Description |
| --- | --- | --- |
| Memory Frequency | [Auto] 800 1000 1066 1200 1333 1400 1600 1800 1866 2000 2133 2400 2600 2666 | Maximum Memory Frequency Selections in Mhz. Do not select Reserved |
| Numa | Disabled [Enabled] | Enable or Disable Non uniform Memory Access (NUMA). |
| Publish SRAT | Disabled [Enabled] | Publish the SRAT ACPI table to the OS. |
| SRAT Memory Hot Plug | [Disabled] Enabled | Fix for OS that does not support memory hot plug. Ex: SuSE SLES10 SP2. Enable by default. Disable will clear all hot plug bits and remove the hot plug entries in SRAT table." |
| Static Virtual Lockstep Mode | [Disabled] Enabled | Enable Static Virtual Lockstep mode |
| Mirror mode | [Disabled] Mirror Mode 1LM Mirror Mode 2LM | Mirror Mode will set entire 1LM/2LM memory in system to be mirrored, consequently reducing the memory capacity by half. Mirror Enable will disable XPT Prefetch |
| Mirror TAD0 | [Disabled] Enabled | Enable Mirror on entire memory for TAD0 |
| Enable Partial Mirror | [Disabled] Partial Mirror mode 1LM Partial Mirror mode 2LM | Partial mirror mode will enable the required size of memory to be mirrored. If rank sparing is enabled partial mirroring will not take effect. Mirror Enable will disable XPT Prefetch |
| UEFI ARM Mirror | [Disabled] Enabled | Imitate behavior of UEFI based Address Range Mirror with setup option |
| Memory Rank Sparing | [Disabled] Enabled | Enable/Disable Memory Rank Sparing |
| Data Scrambling for DDR4 | [Auto] Disabled Enabled | Enable - Enables data scrambling for DDR4. Disable - Disables this feature. Auto - Sets it to the MRC default setting; current default is Enable. |
| Patrol Scrub | Disabled [Enabled] | Enable/Disable Patrol Scrub |
| Patrol Scrub Interval | 24 | Selects the number of hours (1-24) required to complete full scrub. A value of zero means auto! |
| Patrol Scrub Address Mode | Reverse Address [System Physical Address] | Selects the address mode between System Physical Address (or) Reverse Address |

IIO Configuration

| Item | Option | Description |
| --- | --- | --- |
| Intel® VT for Directed I/O (VT-d) | [Disabled] Enabled | Enable/Disable Intel® Virtualization Technology for Directed I/O (VT-d) by reporting the I/O device assignment to VMM through DMAR ACPI Tables. |
| PCI-E ASPM Support (Global) | [Disabled] L1 Only | This option enables / disables the ASPM support for all downstream devices. |
| CPU Socket 0 |  |  |
| PCI-E Port 1A | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 1C | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 2A | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 2C | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 3A | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 3C | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| CPU Socket 1 |  |  |
| PCI-E Port 1A | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 1C | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 2A | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 2B | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 2C | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 2D | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 3A | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |
| PCI-E Port 3C | [Auto] Disabled Enabled | In auto mode the BIOS will remove the EXP port if there is no device or errors on that device and the device is not HP capable. Disable is used to disable the port and hide its CFG space. |

Server Mgmt

| Item | Option | Description |
| --- | --- | --- |
| Power Policy | Always On Always Off Last state | Select ON/OFF for ACPI state after a G3 |
| Power Button | Disabled [Enable] | Select Power Button function behavior |
| BMC Firmware Revision | Display Only | Display BMC version |
| System Event Log |  |  |
| BMC network configuration |  |  |
| View System Event Log |  |  |

System Event Log

| Item | Option | Description |
| --- | --- | --- |
| Enabling/Disabling Options |  |  |
| SEL Components | Disabled [Enabled] | Change this to enable or disable all features of System Event Logging during boot. |
| Erasing Settings |  |  |
| Erase SEL | [No] Yes, On next reset Yes, On every reset | Choose options for erasing SEL. |
| When SEL is Full | [Do Nothing] Erase Immediately | Choose options for reactions to a full SEL. |
| Log EFI Status Codes | Disabled Both [Error code] Progress code | Disable the logging of EFI Status Codes or log only error code or only progress code or both. |

BMC network configuration

| Item | Option | Description |
| --- | --- | --- |
| Configure IPV4 Support |  |  |
| Lan channel 1 |  |  |
| Configuration Address source | [Unspecified] Static DynamicBmcDhcp | Select to configure LAN channel parameters statically or dynamically (by BIOS or BMC). Unspecified option will not modify any BMC network parameters during BIOS phase. When " BMC Network Bonding" is “Enabled”, this item is not visible. |
| Current Configuration Address source | Read only |  |
| Station IP address | Read only |  |
| Subnet mask | Read only |  |
| Station MAC address | Read only |  |
| Router IP address | Read only |  |
| Router MAC address | Read only |  |
| Lan channel 2 |  |  |
| Configuration Address source | [Unspecified] Static DynamicBmcDhcp | Select to configure LAN channel parameters statically or dynamically (by BIOS or BMC). Unspecified option will not modify any BMC network parameters during BIOS phase. When " BMC Network Bonding" is “Enabled”, this item is not visible. |
| Current Configuration Address source | Read only |  |
| Station IP address | Read only |  |
| Subnet mask | Read only |  |
| Station MAC address | Read only |  |
| Router IP address | Read only |  |
| Router MAC address | Read only |  |
| Configure IPV6 Support |  |  |
| Lan channel 1 |  |  |
| IPV6 Support | [Enabled] Disabled |  |
| Configuration Address source | [Unspecified] Static DynamicBmcDhcp |  |
| Current Configuration Address source | Read only |  |
| Station IPV6 address | Read only |  |
| Prefix Length | Read only |  |
| IPV6 Router1 IP Address | Read only |  |
| IPV6 address status | Read only |  |
| IPV6 DHCP Alogorithm | Read only |  |
| Lan channel 2 |  |  |
| IPV6 Support | [Enabled] Disabled |  |
| Configuration Address source | [Unspecified] Static DynamicBmcDhcp |  |
| Current Configuration Address source | Read only |  |
| Station IPV6 address | Read only |  |
| Prefix Length | Read only |  |
| IPV6 Router1 IP Address | Read only |  |
| IPV6 address status | Read only |  |
| IPV6 DHCP Alogorithm | Read only |  |

BMC network configuration

Security

| Item | Option | Description |
| --- | --- | --- |
| Administrator Password |  | Set Administrator Password |
| User Password |  | Set User Password |

Boot

| Item | Option | Description |
| --- | --- | --- |
| Setup Prompt Timeout | 5 | Number of seconds to wait for setup activation key. 65535(0xFFFF) means indefinite waiting. |
| Boot up Num Lock State | [On] Off | Select the keyboard NumLock state |
| Quiet Boot | [Disabled] Enabled | Enables or disables Quiet Boot option |
| Boot mode select | [UEFI] Legacy | Select boot mode LEGACY/UEFI |
| Boot Option Priority |  | Display Boot Option Priority |

Save & Exit

| Item | Option | Description |
| --- | --- | --- |
| Save Changes and Exit |  | Exit the system after saving the changes. |
| Discard Changes and Exit |  | Exit system setup without saving any changes. |
| Save Changes and Reset |  | Reset the system after saving the changes. |
| Discard Changes and Reset |  | Reset system setup without saving any changes. |
| Save Changes |  | Save Changes done so far to any of the setup options. |
| Discard Changes |  | Discard Changes done so far to any of the setup options. |
| Restore Defaults |  | Restore/Load Default values for all the setup options. |
| Save as User Defaults |  | Save the changes done so far as User Defaults. |
| Restore User Defaults |  | Restore the User Defaults to all the setup options. |
| Boot Override |  | Display One-Time Boot Menu options |

Appendix A
	Motherboard connectors and slots

Appendix B 
   Memory Slot Layout

Appendix C	
Memory Module Installation Order

The memory configurations supported by SR205-2 server are:

Independent Mode

Mirror Mode

Sparing Mode

The installation sequences for each mode are demonstrated in the following tables.

Independent Mode

| Number of Processor |  | Installation Sequence |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Processor x 1 | CPU1 | A1 | D1 | B1 | E1 | C1 | F1 | A2 | D2 | B2 | E2 | C2 | F2 | - | - | - | - | - | - | - | - | - | - | - | - |
| Processor x 2 | CPU1 | A1 | - | D1 | - | B1 | - | E1 | - | C1 | - | F1 | - | A2 | - | D2 | - | B2 | - | E2 | - | C2 | - | F2 | - |
|  | CPU2 | - | A1 | - | D1 | - | B1 | - | E1 | - | C1 | - | F1 | - | A2 | - | D2 | - | B2 | - | E2 | - | C2 | - | F2 |

Mirror Mode

| Number of Processor |  | Installation Sequence |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Processor x 1 | CPU1 | A1 | A2 | B1 | B2 | C1 | C2 | D1 | D2 | E1 | E2 | F1 | F2 | - | - | - | - | - | - | - | - | - | - | - | - |
| Processor x 2 | CPU1 | A1 | A2 | B1 | B2 | C1 | C2 | D1 | D2 | E1 | E2 | F1 | F2 | - | - | - | - | - | - | - | - | - | - | - | - |
|  | CPU2 | - | - | - | - | - | - | - | - | - | - | - | - | A1 | A2 | B1 | B2 | C1 | C2 | D1 | D2 | E1 | E2 | F1 | F2 |

Mirroring is supported with 6 or 12 DIMMs per CPU.

Sparing Mode

| Number of Processor |  | Installation Sequence |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Processor x 1 | CPU1 | A1 | D1 | B1 | E1 | C1 | F1 | A2 | D2 | B2 | E2 | C2 | F2 | - | - | - | - | - | - | - | - | - | - | - | - |
| Processor x 2 | CPU1 | A1 | - | D1 | - | B1 | - | E1 | - | C1 | - | F1 | - | A2 | - | D2 | - | B2 | - | E2 | - | C2 | - | F2 | - |
|  | CPU2 | - | A1 | - | D1 | - | B1 | - | E1 | - | C1 | - | F1 | - | A2 | - | D2 | - | B2 | - | E2 | - | C2 |  | F2 |

Appendix D	
BIOS Function Key

Hot Keys

| Item | Function |
| --- | --- |
| Tab | Allows users to exit Quiet Boot Mode. |
| F1 | Allows users to enter setup. The message “Entering Setup …” will appear during the POST process after pressing the <F1> key. |
| F12 | Allows users to prompt boot menu. The message “Entering Boot Menu …” will appear during the POST process after pressing the <F12> key. |

Keys for BIOS Menu Navigation

| Item | Function |
| --- | --- |
| Up & Down Arrow Keys | Allows users to move the cursor up and down. |
| Left & Right Arrow Keys | Allow users to move to another menu. |
| Enter | Allows users to select an item or enter a sub menu. |
| +/- | Allows users to add or reduce value of the parameters. |
| ESC | Exit |
| F1 | General Help |
| F2 | Previous Value |
| F3 | Optimized Defaults |
| F4 | Save & Exit Setup |

Appendix E	
LED Descriptions

The following LEDs and buttons are located on the front and rear panel and for each individual node:

| Icon | LED/Button Name | LED/Button Definition |
| --- | --- | --- |
|  | Power LED/Button | Button: Press to power the system on/off Press and hold for 4 seconds to force the system to power off LED: Off: System is powered off Blue: System is powered on (Front) Green: System is powered on (Rear) |
|  | UID LED/Button | Button: Press to toggle the ID LED Press and hold for 4 seconds to reset the Baseboard Management Controller (BMC)* LED: Off: System is not identified Blue: System is identified Blinking Blue: System is resetting the BMC *The Baseboard Management Controller is a processor which measures the internal physical variables of the server. |
|  | System Health LED (Front panel) | Red: System error detected Possible problem: Sensor reading over critical threshold CPU Catastrophic Error (CATERR) CPU / Memory VR fault CPU / PCH / DIMM thermal trip Power supply AC lost Power supply failure Blinking Red: System error detected Possible problem: Sensor reading over non-critical threshold Power supply mismatched, or not present System event log (SEL) almost full asserted IPMI watchdog asserted CPU throttling LAN leash lost HDD fault Off: System health is good |

Hard Drive LEDs

The following LEDs are located on the front panel and on each individual hard drive:

| Icon | LED Name | LED Definition |
| --- | --- | --- |
|  | Hard Drive Activity LED (Amber) | Amber: Present Blinking Amber: Locate, activity Off: Not present |
|  | Hard Drive Health LED (Red) | Red: Fail Blinking Red: Locate, rebuild Off: N/A |